Slabs according the Pieper-Martens method.

# INSTALLATION

Unzip the file plakes2008.zip to any folder.

Double click on slabs1.exe for the english version of the software

or  plates4.exe for the greek version of the software.

Find also the file slab.pdf  with a full legend of symbols and notation,

instructions for the software usage and an example.

(Greek version of the above in files plaka.pdf and plakded.pdf)

Find also example.pla the  data file of example and plates.ico

for creating shortcut icons.

# THEORY

Method valid if live load/total load <=2/3, (Q/G+Q <=2/3)

For the 4-way slabs the tables of Pieper-Martens are used (modified Czerny tables to take into account the most unfavourable positions of the live load in a system of adjacent slabs).

The Pieper Martens method results in increased slab moments compared to the moments calculated with the Czerny tables.

For the 3-way and 2-way slabs I have modified the Hahn tables with the same method and philosophy that the Czerny tables, for the 4-way slabs, where modified to Pieper-Martens tables.

Again the modified tables for the 3-way and 2-way slabs result in increased moments compared to the Hahn tables.

# USING THE SOFTWARE

The user has the flexibility to chose a "safer" type of slab if the adjacent slab do not provide sufficient end fixing moment, e.g. a small cantilever, or a slab in continuation with only a part of the slabs edge. See slab S4 of the example.

For the description of the supports a description like 10-10 X or 3-3 Y is valid in case that the user takes advantage of symmetry in a configuration of continues slabs or we have a cantilever

supported on a beam or a wall. The description of the supports is also valid if one of the

adjacent slabs is declared to have both edges simply supported in the direction of continuity. In that case the software calculates as a support moment the support moment of the slab

that has such a moment.

E.G. Slab1 is given as type 1 (for safety reasons) and slab2 is declared as type 4(it is continues with only a part of slab1) then support 1-2 X will have Ms1=Msx1=0, Ms2=Msx2=moment

then the software calculates Msfin=moment.

In the case of continuous slabs where at least on slab is of types 49-59( cantilever slabs) or  of  types 61-72 (2-way slabs), then the final Msfin of such supports have, at least, the value Msx/Msy of the cantilever or 2-way slab.

In general the program is free from the real slab system topology and the user has the flexibility to model the slab system as he judges safer. For example a slab can be given twice (with another

number of course)with different TYPES and the user can take the span reinforcement from one type and the support reinforcement  from the other type.

The above apply also when we have slab shapes that are not rectangular.

e.g. For an L shape slab we can model it as 2 different slabs of TYPES that are "independent" from each other and provide reinforcement for each slab enough to support them without the synergy of the other.

# REINFORCEMENT

The program calculates the reinforcement according to EC2, very similar to Greek and Cypriot codes.

Calculated reinforcement:

For the spans for both directions -X- and -Y-: As1x, As1y.

For the defined supports (common supports of continuous slabs): As2

The user must provide all the auxiliary reinforcement required by the code like the upper reinforcement at the simply supported edges, the distribution reinforcement for the upper reinforcement at the continuous slabs etc.

Feel free to contact with me: santamas@spidernet.com.cy

or visit our site: www.santamas.org

and get also the winkler foundation beam freeware.

Costas Santamas

Civil Engineer.

Nicosia, Cyprus